18071

Q. Demonstrate the RIP Routing in Packet Tracer.

Aim : To write a code to demonstrate RIP routing in packet tracer.

Steps: 
1.In packet tracer take 2 PCs and 2 Routers.

2.Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 	with PC1 via fasethernet0/0 using cross-copper cord.

3.Connect both router via fastethernet0/1 using cross-copper cord.

4.Set the IP address and default gateway of both the PCs in ip configuration of both 	the PCs respectively.

Topology:

Router0: 
Router> enable 
Router# configure terminal 
Router(config)# interface fastethernet0/0 
Router(config-if)# ip address 192.168.1.1 255.255.255.0

| Khushi Vishwakarma | S.I.W.S College | Major Practical 2 |
| --- | --- | --- |

18071

Router(config-if)# no shutdown 
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1 
Router(config-if)# ip address 10.0.0.1 255.255.255.0 Router(config-if)# no shutdown 
Router(config-if)# exit 
Router(config)# router rip 
Router(config-router)# version 2 
Router(config-router)# network 192.168.1.0 
Router(config-router)# network 10.0.0.0 
Router(config-router)# no auto-summary 
Router(config-router)# exit

Router(config)# end 
Router1: 
Router> enable 
Router# configure terminal 
Router(config)# interface gigabitEthernet0/0 
Router(config-if)# ip address 10.0.0.2 255.255.255.0 Router(config-if)# no shutdown 
Router(config-if)# exit

Router(config)# interface gigabitEthernet0/1 
Router(config-if)# ip address 192.168.2.1 255.255.255.0 Router(config-if)# no shutdown 
Router(config-if)# exit 
Router(config)# router rip 
Router(config-router)# version 2 
Router(config-router)# network 10.0.0.0

| Khushi Vishwakarma | S.I.W.S College | Major Practical 2 |
| --- | --- | --- |

18071

Router(config-router)# network 192.168.2.0 
Router(config-router)# no auto-summary 
Router(config-router)# exit

Router(config)# end 
Router#show ip route 
Router#show ip protocol 
Output:

Verification:In PC0 command prompt.

C:>ipconfig 
C:\>ping 192.168.2.2 
Output:

| Khushi Vishwakarma | S.I.W.S College | Major Practical 2 |
| --- | --- | --- |

18071

Conclusion:The abovr topology has been executed successfully.

| Khushi Vishwakarma | S.I.W.S College | Major Practical 2 |
| --- | --- | --- |