18071

Q. Demonstrate the Static Routing in Packet Tracer.

Aim : To write a code to demonstrate static routing in packet tracer.

Steps: 
1.In packet tracer take 2 PCs and 2 Routers.

2.Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 	with PC1 via fasethernet0/0 using cross-copper cord.

3.Connect both router via fastethernet0/1 using cross-copper cord.

4.Set the IP address and default gateway of both the PCs in ip configuration of both 	the PCs respectively.

Topology:

Code: 
Router0: 
Router> enable 
Router# configure terminal

| Khushi vishwakarma | S.I.W.S College | Magor Practical 2 |
| --- | --- | --- |

18071

Router(config)# interface fastethernet0/0 
Router(config-if)# ip address 10.0.0.1 255.0.0.0 Router(config-if)# no shutdown 
Router(config-if)# exit 
Router(config)# interface fastethernet0/1 
Router(config-if)# ip address 20.0.0.1 255.0.0.0 Router(config-if)# no shutdown 
Router(config-if)# exit 
Router(config)#ip route 40.0.0.0 255.0.0.0 20.0.0.2 Router(config)#exit 
Router1: 
Router> enable 
Router# configure terminal 
Router(config)# interface fastethernet0/0 
Router(config-if)# ip address 20.0.0.2 255.0.0.0 Router(config-if)# no shutdown 
Router(config-if)# exit 
Router(config)# interface fastethernet0/1 
Router(config-if)# ip address 40.0.0.1 255.0.0.0 Router(config-if)# no shutdown 
Router(config-if)# exit 
Router(config)#ip route 10.0.0.0 255.0.0.0 20.0.0.1 Router(config)#exit

Verification: In Both  PCs Command Prompt.

| Khushi vishwakarma | S.I.W.S College | Magor Practical 2 |
| --- | --- | --- |

18071

PC0: 
C:\ipconfig 
C:\ping 10.0.0.1 
C:\ping 10.0.0.2 
C:\ping 20.0.0.1 
C:\ping 20.0.0.2 
C:\ping 40.0.0.1 
C\:ping 40.0.0.2 
PC1: 
 C:\ipconfig 
C:ping 40.0.0.1 
C:ping 20.0.0.2 
C:ping 20.0.0.1 
C:ping 10.0.0.1 
C:ping 10.0.0.2

Output: 
PC0:

| Khushi vishwakarma | S.I.W.S College | Magor Practical 2 |
| --- | --- | --- |

18071

PC1:

Conclusion:The above Topology has been executed successfully.

| Khushi vishwakarma | S.I.W.S College | Magor Practical 2 |
| --- | --- | --- |

18071

| Khushi vishwakarma | S.I.W.S College | Magor Practical 2 |
| --- | --- | --- |