18071

Q. Demonstrate Routing OSPF in packet tracer.

Aim: To write a code to demonstrate routing OSPF in packet tracer.

Steps: 
 1. In packet tracer take 2 PCs and 2 Routers.

2. Connect Router0 with PC0 via fastethernet0/0 using cross-copper cord and Router1 with PC1 via fasethernet0/0 using cross-copper cord.

3. Connect both router via fastethernet0/1 using cross-copper cord.

4. Set the IP address and default gateway of both the PCs in ip configuration of both the PCs respectively.

Topology:

Code: 
Router0: 
 Router> enable 
 Router# configure terminal 
Router(config)# interface fastethernet0/0

| Khushi Vishwakarma | S.I.W.S College | Major Practical 2 |
| --- | --- | --- |

18071

Router(config-if)# ip address 192.168.1.1 255.255.255.0 Router(config-if)# no shutdown 
 Router(config-if)# exit 
 Router(config)# interface fastethernet0/1 
 Router(config-if)# ip address 10.0.0.1 255.255.255.0 
 Router(config-if)# no shutdown 
 Router(config-if)# exit 
Router(config)#router ospf 1 
Router(config-router)#network 192.168.1.0 0.0.0.255 area 0 Router(config-router)#exit 
Router1: 
Router> enable 
 Router# configure terminal 
Router(config)# interface fastethernet0/0 
Router(config-if)# ip address 10.0.0.2 255.255.255.0 
Router(config-if)# no shutdown 
 Router(config-if)# exit 
 Router(config)# interface fastethernet0/1 
 Router(config-if)# ip address  192.168.2.1 255.255.255.0  Router(config-if)# no shutdown 
 Router(config-if)# exit 
Router(config)#router ospf 2 
Router(config-router)#network 10.0.0.0 0.0.0.255 area 0 Router(config-router)#network 192.168.2.0 0.0.0.255 area 0 Router(config-router)#exit 
Verification :In PC in command Prompt.

| Khushi Vishwakarma | S.I.W.S College | Major Practical 2 |
| --- | --- | --- |

18071

PC0: 
C:\ipconfig 
C:\ping 192.168.2.10 
Output:

Verification: On any one Router 
Router#show ip ospf neighbor 
Router#show ip protocol 
Router#show ip protocol 
Output:

Conclusion : The above Topology has been executed successfully.

| Khushi Vishwakarma | S.I.W.S College | Major Practical 2 |
| --- | --- | --- |

18071

| Khushi Vishwakarma | S.I.W.S College | Major Practical 2 |
| --- | --- | --- |